Without casing

Includes waterproof case

[Features]

- Supports 4G LTE, 3G, and 2G; multi-band support ensures stable

global connectivity

- Features an automatic failover mechanism to ensure

continuous communication if the primary connection is interrupted.

- Features a rugged housing and wide-temperature design,

suitable for various harsh industrial environments.

- Built-in digital I/O supports remote device monitoring

and automated control.

- Provides comprehensive security protection and multiple VPN encryption mechanisms to ensure data security.

- Supports cloud-based remote management, significantly improving maintenance efficiency and reducing costs.

- Supports Wi-Fi functionality, serving as a wireless access point for multiple devices to connect.

| 4GR | ━ | Code 1 | ━ | Code 2 |
| --- | --- | --- | --- | --- |

[Optional Code]

| Code 1 | Number of ethernet holes | Code 2 | Shell |
| --- | --- | --- | --- |
| 1 | WAN*1 and LAN*1 can be adjusted to LAN*2 | N | None |
| 4 | 4 LAN | 175 | IP68 waterproof case (PCW175) |
|  |  | ABS | IP68 waterproof case 300*300*180mm |

※If code 1 is 4 LAN, code 2 must be equipped with ABS.
※4G router compatible with unlimited data SIM plans only.

4GR

(4G SIM card is required for Internet access)

Camera

Computer

[Wiring Application Diagram]

[Specifications]

| Power supply | 9-30 VDC，Power Consumption <6.5W |  |
| --- | --- | --- |
| LTE Module | Supports 4G/LTE (Cat 4), 3G and 2G networks |  |
| Processor and memory | CPU | Mediatek MIPS 24Kc 580MHz |
|  | Memory | 128 MB RAM |
|  | Storage space | 16 MB Flash |
| Interface and Slots | Ethernet | 2 x 10/100 ports (1 x WAN configurable as LAN, 1 x LAN) |
|  | SIM 卡 | 1 external SIM card slot |
|  | Digital I/O | One digit input, one digit output |
| Wireless Functionality | With WiFi access point and hotspot functionality. |  |
| Operating Temperature | -40 °C to 75 °C |  |
| Network Protocols | Supports TCP, UDP, IPv4, IPv6, ICMP, DNS, HTTP/HTTPS |  |
| Firewall | Protection against DDoS attacks (such as SYN flood) and port scanning |  |